# Lab - Build a Switch and Router Network

# Topology

# Addressing Table

| Device | Interface | IP Address / Prefix | Default Gateway |
| --- | --- | --- | --- |
| R1 | G0/0/0 | 192.168.0.1 /24 | N/A |
| R1 | G0/0/0 | 2001:db8:acad::1/64 | N/A |
| R1 | G0/0/0 | fe80::1 | N/A |
| R1 | G0/0/1 | 192.168.1.1 /24 | N/A |
| R1 | G0/0/1 | 2001:db8:acad:1::1/64 | N/A |
| R1 | G0/0/1 | fe80::1 | N/A |
| S1 | VLAN 1 | 192.168.1.2 /24 | 192.168.1.1 |
| PC-A | NIC | 192.168.1.3 /24 | 192.168.1.1 |
| PC-A | NIC | 2001:db8:acad:1::3/64 | fe80::1 |
| PC-B | NIC | 192.168.0.3 /24 | 192.168.0.1 |
| PC-B | NIC | 2001:db8:acad::3/64 | fe80::1 |

# Objectives

Part 1: Set Up the Topology and Initialize Devices

Part 2: Configure Devices and Verify Connectivity

# Background / Scenario

This is a comprehensive lab to review previously covered IOS commands. In this lab, you will cable the equipment as shown in the topology diagram. You will then configure the devices to match the addressing table. After the configurations have been saved, you will verify your configurations by testing for network connectivity.

After the devices have been configured and network connectivity has been verified, you will use IOS commands to retrieve information from the devices to answer questions about your network equipment.

This lab provides minimal assistance with the actual commands necessary to configure the router. Test your knowledge by trying to configure the devices without referring to the content or previous activities.

Note: The routers used with CCNA hands-on labs are Cisco 4221 with Cisco IOS XE Release 16.9.4 (universalk9 image). The switches used in the labs are Cisco Catalyst 2960s with Cisco IOS Release 15.2(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of the lab for the correct interface identifiers.

Note: Ensure that the routers and switches have been erased and have no startup configurations. Consult with your instructor for the procedure to initialize and reload a router and switch.

The default bias template used by the Switch Database Manager (SDM) does not provide IPv6 address capabilities. Verify that SDM is using either the dual-ipv4-and-ipv6 template or the lanbase-routing template. The new template will be used after reboot even if the configuration is not saved.

S1# show sdm prefer

Use the following commands to assign the dual-ipv4-and-ipv6 template as the default SDM template.

S1# configure terminal

S1(config)# sdm prefer dual-ipv4-and-ipv6 default

S1(config)# end

S1# reload

# Required Resources

1 Router (Cisco 4221 with Cisco IOS XE Release 16.9.4 universal image or comparable)

1 Switch (Cisco 2960 with Cisco IOS Release 15.2(2) lanbasek9 image or comparable)

2 PCs (Windows with a terminal emulation program, such as Tera Term)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Note: The Gigabit Ethernet interfaces on Cisco 4221 routers are autosensing and an Ethernet straight-through cable may be used between the router and PC-B. If using another model Cisco router, it may be necessary to use an Ethernet crossover cable.

# Instructions

## Set Up Topology and Initialize Devices

### Cable the network as shown in the topology.

Attach the devices shown in the topology diagram, and cable, as necessary.

Power on all the devices in the topology.

### Initialize and reload the router and switch.

If configuration files were previously saved on the router and switch, initialize and reload these devices back to their default configurations.

## Configure Devices and Verify Connectivity

In Part 2, you will set up the network topology and configure basic settings, such as the interface IP addresses, device access, and passwords. Refer to the Error! Reference source not found. and Error! Reference source not found. at the beginning of this lab for device names and address information.

### Assign static IP information to the PC interfaces.

Configure the IP address, subnet mask, and default gateway settings on PC-A.

Configure the IP address, subnet mask, and default gateway settings on PC-B.

Ping PC-B from a command prompt window on PC-A.

Note: If pings are not successful, the Windows Firewall may need to be turned off.

#### Question:

Why were the pings not successful?

Type your answers here.

### Configure the router.

Console into the router and enable privileged EXEC mode.

Open configuration window

Enter configuration mode.

Assign a device name to the router.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Assign cisco as the VTY password and enable login.

Encrypt the plaintext passwords.

Create a banner that warns anyone accessing the device that unauthorized access is prohibited.

Configure and activate both interfaces on the router.

Configure an interface description for each interface indicating which device is connected to it.

To enable IPv6 routing, enter the command ipv6 unicast-routing.

R1(config)# ipv6 unicast-routing

Save the running configuration to the startup configuration file.

Set the clock on the router.

Note: Use the question mark (?) to help with the correct sequence of parameters needed to execute this command.

Close configuration window

Ping PC-B from a command prompt window on PC-A.

Note: If pings are not successful, the Windows Firewall may need to be turned off.

#### Question:

Were the pings successful? Explain.

Type your answers here.

### Configure the switch.

In this step, you will configure the hostname, the VLAN 1 interface and its default gateway.

Open configuration window

Console into the switch and enable privileged EXEC mode.

Enter configuration mode.

Assign a device name to the switch.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

Configure and activate the VLAN interface on the switch S1.

Configure the default gateway for the switch S1.

Save the running configuration to the startup configuration file.

### Verify connectivity end-to-end connectivity.

From PC-A, ping PC-B.

From S1, ping PC-B.

All the pings should be successful.

Close configuration window

## Display Device Information

In Part 3, you will use show commands to retrieve interface and routing information from the router and switch.

### Display the routing table on the router.

Use the show ip route command on the router R1 to answer the following questions.

Open configuration window

#### Questions:

What code is used in the routing table to indicate a directly connected network?

Type your answers here.

How many route entries are coded with a C code in the routing table?

Type your answers here.

What interface types are associated to the C coded routes?

Type your answers here.

Use the show ipv6 route command on router R1 to display the IPv6 routes.

### Display interface information on the router R1.

Use the show ip interface g0/0/1 to answer the following questions.

#### Questions:

What is the operational status of the G0/0/1 interface?

Type your answers here.

What is the Media Access Control (MAC) address of the G0/1 interface?

Type your answers here.

How is the Internet address displayed in this command?

Type your answers here.

For the IPv6 information, enter the show ipv6 interface interface command.

### Display a summary list of the interfaces on the router and switch.

There are several commands that can be used to verify an interface configuration. One of the most useful of these is the show ip interface brief command. The command output displays a summary list of the interfaces on the device and provides immediate feedback to the status of each interface.

Enter the show ip interface brief command on the router R1.

R1# show ip interface brief

To see the IPv6 interface information, enter the show ipv6 interface brief command on R1.

R1# show ipv6 interface brief

Close configuration window

Enter the show ip interface brief command on the switch S1.

Open configuration window

S1# show ip interface brief

Close configuration window

# Reflection Questions

If the G0/0/1 interface showed that it was administratively down, what interface configuration command would you use to turn the interface up?

Type your answers here.

What would happen if you had incorrectly configured interface G0/0/1 on the router with an IP address of 192.168.1.2?

Type your answers here.

# Router Interface Summary Table

| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| --- | --- | --- | --- | --- |
| 1800 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 1900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2801 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 2811 | Fast Ethernet 0/0 (F0/0) | Fast Ethernet 0/1 (F0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 4221 | Gigabit Ethernet 0/0/0 (G0/0/0) | Gigabit Ethernet 0/0/1 (G0/0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 4300 | Gigabit Ethernet 0/0/0 (G0/0/0) | Gigabit Ethernet 0/0/1 (G0/0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |

Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many interfaces the router has. There is no way to effectively list all the combinations of configurations for each router class. This table includes identifiers for the possible combinations of Ethernet and Serial interfaces in the device. The table does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface.

End of Document

# Lab - Build a Switch and Router Network

# Topology

# Addressing Table

| Device | Interface | IPv6 Address | Prefix Length | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 2001:db8:acad:a::1 | 64 | N/A |
| R1 | G0/0/1 | 2001:db8:acad:1::1 | 64 | N/A |
| S1 | VLAN 1 | 2001:db8:acad:1::b | 64 | N/A |
| PC-A | NIC | 2001:db8:acad:1::3 | 64 | fe80::1 |
| PC-B | NIC | 2001:db8:acad:a::3 | 64 | fe80::1 |

Blank Line, No additional information

# Objectives

Part 1: Set Up Topology and Configure Basic Router and Switch Settings

Part 2: Configure IPv6 Addresses Manually

Part 3: Verify End-to-End Connectivity

# Background / Scenario

In this lab, you will configure hosts and device interfaces with IPv6 addresses.  You will issue show commands to view IPv6 unicast addresses. You will also verify end-to-end connectivity using ping and traceroute commands.

Note: The routers used with CCNA hands-on labs are Cisco 4221 with Cisco IOS XE Release 16.9.4 (universalk9 image). The switches used in the labs are Cisco Catalyst 2960s with Cisco IOS Release 15.2(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of the lab for the correct interface identifiers.

Note: Make sure that the routers and switches have been erased and have no startup configurations. If you are unsure, contact your instructor.

Note: The default 2960 Switch Database Manager (SDM) template does not support IPv6. It may be necessary to issue the command sdm prefer dual-ipv4-and-ipv6 default to enable IPv6 addressing before applying an IPv6 address to the VLAN 1 SVI.

Note: The default bias template used by the Switch Database Manager (SDM) does not provide IPv6 address capabilities. Verify that SDM is using either the dual-ipv4-and-ipv6 template or the lanbase-routing template. The new template will be used after reboot.

S1# show sdm prefer

Follow these steps to assign the dual-ipv4-and-ipv6 template as the default SDM template:

S1# configure terminal

S1(config)# sdm prefer dual-ipv4-and-ipv6 default

S1(config)# end

S1# reload

# Required Resources

1 Router (Cisco 4221 with Cisco IOS XE Release 16.9.4 universal image or comparable)

1 Switch (Cisco 2960 with Cisco IOS Release 15.2(2) lanbasek9 image or comparable)

2 PCs (Windows with terminal emulation program, such as Tera Term)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Note: The Gigabit Ethernet interfaces on Cisco 4221 routers are autosensing and an Ethernet straight-through cable may be used between the router and PC-B. If using another model Cisco router, it may be necessary to use an Ethernet crossover cable.

# Instructions

## Cable the Network and Configure Basic Router and Switch Settings

After cabling the network, initializing and reloading the router and switch, complete the following:

### Configure the router.

Assign the hostname and configure basic device settings.

### Configure the switch.

Assign the hostname and configure basic device settings.

## Configure IPv6 Addresses Manually

### Assign the IPv6 addresses to Ethernet interfaces on R1.

Assign the IPv6 global unicast addresses, listed in the Addressing Table, to both Ethernet interfaces on R1.

Open configuration window

Verify that the correct IPv6 unicast address is assigned to each interface.

Note: The link-local address (fe80::) displayed is based on EUI-64 addressing, which automatically uses the interface Media Access Control (MAC) address to create a 128-bit IPv6 link-local address.

To get the link-local address to match the global unicast address on the interface, manually enter the link-local addresses on each of the Ethernet interfaces on R1.

Note: Each router interface belongs to a separate network. Packets with a link-local address never leave the local network; therefore, you can use the same link-local address on both interfaces.

Use a command of your choice to verify that the link-local address has been changed to fe80::1.

Close configuration window

#### Question:

Which two multicast groups have been assigned to interface G0/0/0?

Type your answers here.

### Enable IPv6 routing on R1.

On a PC-B command prompt, enter the ipconfig command to examine IPv6 address information assigned to the PC interface.

#### Question:

Has an IPv6 unicast address been assigned to the network interface card (NIC) on PC-B?

Type your answers here.

Enable IPv6 routing on R1 using the IPv6 unicast-routing command.

Open configuration window

Use a command to verify the new multicast group are assigned to interface G0/0/0. Notice that the all-router multicast group (ff02::2) now appears for interface G0/0/0.

Note: This will allow the PCs to obtain their IP address and default gateway information automatically using Stateless Address Autoconfiguration (SLAAC).

Now that R1 is part of the all-router multicast group FF02::2, re-issue the ipconfig command on PC-B and examine the IPv6 address information.

#### Question:

Why did PC-B receive the Global Routing Prefix and Subnet ID that you configured on R1?

Type your answers here.

### Assign IPv6 addresses to the management interface (SVI) on S1.

Assign the IPv6 address for S1. Also assign a link-local address for this interface.

Use a command of your choice to verify that the IPv6 addresses are properly assigned to the management interface.

Close configuration window

### Assign static IPv6 addresses to the PCs.

Open the Ethernet Properties window on for each PC and assign IPv6 addressing.

Verify both PCs have the correct IPv6 address information. Each PC should have two Global IPv6 addresses: one static and one SLACC

## Verify End-to-End Connectivity

    - From PC-A, ping fe80::1. This is the link-local address assigned to G0/0/1 on R1.

    - Ping the S1 management interface from PC-A.

    - Use the tracert command on PC-A to verify that you have end-to-end connectivity to PC-B.

    - From PC-B, ping PC-A.

    - From PC-B, ping the link-local address for G0/0/0 on R1.

Note: If end-to-end connectivity is not established, troubleshoot your IPv6 address assignments to verify that you entered the addresses correctly on all devices.

# Reflection Questions

Why can the same link-local address, fe80::1, be assigned to both Ethernet interfaces on R1?

Type your answers here.

What is the Subnet ID of the IPv6 unicast address 2001:db8:acad::aaaa:1234/64?

Type your answers here.

# Router Interface Summary Table

| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| --- | --- | --- | --- | --- |
| 1900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 2900 | Gigabit Ethernet 0/0 (G0/0) | Gigabit Ethernet 0/1 (G0/1) | Serial 0/0/0 (S0/0/0) | Serial 0/0/1 (S0/0/1) |
| 4221 | Gigabit Ethernet 0/0/0 (G0/0/0) | Gigabit Ethernet 0/0/1 (G0/0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |
| 4300 | Gigabit Ethernet 0/0/0 (G0/0/0) | Gigabit Ethernet 0/0/1 (G0/0/1) | Serial 0/1/0 (S0/1/0) | Serial 0/1/1 (S0/1/1) |

Blank Line, No additional information

Note: To find out how the router is configured, look at the interfaces to identify the type of router and how many interfaces the router has. There is no way to effectively list all the combinations of configurations for each router class. This table includes identifiers for the possible combinations of Ethernet and Serial interfaces in the device. The table does not include any other type of interface, even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in Cisco IOS commands to represent the interface.

End of document